here’s what we added/changed to make ST1, ST2, ST3 actually work with the PLC (in simple terms):

1) we fixed the network “wiring” (ports/handles)

- each station must listen/receive on its own station port (ST1=6001, ST2=6002, ST3=6003).

- we stopped using the wrong / shared socket handles.

- we made sure send/recv uses the station’s real connected handle/port, not a default PLC handle.

- result: stations finally receive the PLC command packet and can reply back with status.

2) we unified the packet format (command + status)

we added/used a consistent encode/decode for:

- PLC → Station command: cmd_start, cmd_stop, cmd_reset, batch_id, recipe_id

- Station → PLC status: ready, busy, fault, done, cycle_time_ms + station KPIs

so the PLC and station agree on what “9 bytes” (or whatever length) means.

3) we added the handshake pattern (this was the big one)

we implemented the same safe pattern in the stations:

a) “start latch” (Latched Enable)

- station does NOT run just because it’s alive.

- it runs only after it sees cmd_start=True.

- we added a flag like: start_latched

- once latched, the station keeps running even if the PLC command packet repeats.

this stops random auto-start and makes the behavior stable.

b) “one-shot done”

- when a cycle finishes, we raise done=1 for one cycle/one report

- then we hold/clear it safely (often using done_latched)

- and we wait for PLC behavior like “cmd_start goes low” or “cmd_reset” to fully re-arm the next cycle

this prevents “done stuck high forever” or “done spam”.

4) we added the station runtime step gating (should_step)

each station now does this every tick:

- receive latest PLC command

- decide:

  - if start_latched is false → should_step=False → don’t advance SimPy logic

  - if latched → should_step=True → advance the process/state machine

- send status back to PLC

that’s why your logs show:

- start_latched=False, busy=False, should_step=False
until cmd_start=True.

5) we added clearer state + logs (so we could debug)

we added prints like:

- “received command packet”

- “decoded PLC command…”

- “env.now, dt_s, start_latched, busy, should_step”

- “sending to PLC on port …”

that’s how we proved the station is receiving commands, not guessing.

quick summary in one line

we made ST1/ST2/ST3 work by fixing who talks to who (ports/handles), then enforcing a real PLC handshake using start_latched + one-shot done, and only letting the station process run when cmd_start is actually true.

if you paste the PLC log lines where it decides S3_cmd_start (or the part of PLC state machine), i can tell you why PLC is still sending cmd_start=False for ST3/ST4 in your screenshot.

1) the “generator”

in our code, a generator is just a python function that uses yield.

why we used it:

- SimPy processes are generators. anything like def run(): ... yield env.timeout(...) is a generator.

- we added/used a main loop (often called something like SimRuntime / station_loop / run()) that:

  - receives the PLC command

  - updates start_latched / should_step

  - advances the station state only if allowed

  - sends status back

  - yield env.timeout(dt) to move simulation time forward

without that generator loop, the station either:

- runs once then stops, or

- blocks and never gives time back to the simulator, or

- can’t “tick” while waiting for PLC commands.

so when you saw logs like env.now=0.000s dt=0.1, that’s the generator loop ticking.

2) the “buffer”

the buffer is the “in-between storage” between stations.

why we added it:

- if ST2 starts while ST1 hasn’t produced anything yet, the pipeline is fake.

- buffer makes it real: ST1 produces → buffer increases → ST2 consumes → buffer decreases.

how it’s usually implemented in our line:

- simplest: a counter in the PLC (or coordinator)

  - buf_12 += 1 when ST1 done one-shot

  - PLC only allows ST2 start when buf_12 > 0

  - when ST2 actually starts, buf_12 -= 1

- more detailed: simpy.Store() or simpy.Container() (queue of tokens/parts)

what it fixes:

- stops downstream stations from running “empty”

- stops multiple starts for one part

- makes throughput + WIP measurable

quick mental model

- generator = time + steps

- buffer = material flow between stations